==============================================================================

Zac Schroff's VGASizes collection : A group of standard VGA font loaders

configured for various screen dimensions.

==============================================================================

This is a simplistic little collection of `hacked together' VGA text size

setting routines.  These routines should work on any true register compatible

VGA (or SVGA) board, but no guarantees are made.  The risk of monitor damage

due to increased stress is minimal, because all video parameters used by these

routines are used by the VGA for at least one of its standard modes.  Even

though this is the case, by using these routines, you agree that the author is

not to be held liable for any damage to hardware or sofrware incurred by using

them.  No warranties of any kind are made.

This package is freeware.  You are encouraged to use it, and in this case,

learn from it.  If have a comment or suggestion, feel welcome to write the

author about it.

Zac Schroff

2906 Firethorn Drive

Tusc AL 35405 USA

==============================================================================

Information on VGA mode and font setting is *VERY* difficult to come by in

any really useful form.  These routines are all the most simplistic methodry I

could come up with to accomplish the task, and reverse engineering is actually

encouraged (they were written using DEBUG, so this should be easy).

Here is a table listing the screen lines for each font used, and what it

produces for screen dimensions when used with each different raster count.

Note the horizontal does not require a table -- horizontal width is changed by

changing the clocking frequency, so both 40 and 80 column modes will use the

same font.

Text lines produced with various fonts

Logical   Physical  ---------------------------------------

Rasters   Rasters   8*8 font       8*14 font      8*16 font

--------  --------  --------       ---------      ---------

175       350       21a            12a            10

200       400       25a            14             12b

240       480       30a            18             15

350       350       43             25b            21b

400       400       50             28             25c

480       480       60             34             30b

Note that there are several font and raster combinations which provide the

same dimensions.  These also have a letter.  The closer the letter to `z', the

higher the resolution for that mode.  The closer the letter to `a', the lower

the resolution.

==============================================================================

Known bugs include the fact that most of these routines will not work with

EGA or MCGA (they will produce unintended results).

Earlier versions of 12.COM, 14.COM and 25A.COM tended to only work on VGAs

with a colour monitor.  This has been fixed (I think).

==============================================================================